Architectural and Engineering Specifications For The Baxall ICE Range Of Closed Circuit Television Dome Cameras

MonOCHROME DOME Cameras	2

1.1	1/3" High Resolution B/W Dome Camera - ICED-B3H39	2

COLOUR/Mono SWITCHiNG DOME Cameras	3

1.2	1/3" High Resolution Col/Mon Dome Camera – ICED-CM3H39	3

1.3	1/3" Ultra Resolution Col/Mon Dome Camera – ICED-CM3U39	4

DAY/NIGHT Switching DOME cameras	5

1.4	1/3" Ultra Res Day/Night Switching Dome Camera – ICED-DN3U39….…...5

WIDE DYNAMIC RANGE DOME cameras	6

1.5	Wide Dynamic Colour Dome Camera – ICED-HYPERDOME	6

## Monochrome Dome Cameras

## 1/3” High Resolution B/W Dome Camera - ICED-B3H39

### The camera shall be a 1/3” monochrome SuperHAD™ CCD.

### It shall provide a horizontal resolution of 570 Television lines / picture height.

### It shall be capable of providing an image at a scene illumination of 0.07 lux, using an F1.2 lens with no bubble.

### The dome shall have a 3-axis GyroView set up.

### It shall be Surface or Ceiling mountable.

### The dome shall have a removable shroud for ceiling mount purposes. Once the shroud has been removed, the dome should be secured via the integral fixing clamps.

### Fixing screws should be provided to allow the dome to be surface mounted.

### The camera shall have a signal to noise ratio of better than 50dB under normal operating conditions and no additional gain.

### The camera shall have an integrated 3~9mm Varifocal lens with DC iris.

### The Electronic Iris on the camera shall have the facility to vary the shutter speed from 1/50 to 1/100,000 of a second, to enable fast capture of moving objects.

### The camera shall have settings for Back Light Compensation (BLC) - On/Off and centre weighted - and Automatic Gain Control (AGC) – On/Off, max <28dB.

### The camera shall have gamma correction of 0.45.

### The camera shall provide a standard CCIR composite video signal of 1v pk-pk when terminated with 75. Connection shall be via a 75 BNC connector.

### The camera shall have an extra video output on the front for a service connector. This optional service connector will be called ICED-SERV.

### The camera shall be capable of line locking to the external low voltage AC power supply. It shall also be capable of phase correction of ±120° using a potentiometer. The camera shall also have the facility to be internally synchronised.

### The camera shall operate when powered by either 12V DC (±10%) or 24V AC (±15%). Power consumption shall be less than 3.4 Watts.

### All settings should be situated adjacent to the lens.

### The camera shall be a Baxall ICED-B3H39.

# Colour/Monochrome Dome Cameras

## 1/3” High Resolution Colour/Mono Dome Camera -ICED-CM3H39

### The camera shall be a 1/3” SuperHAD™ CCD.

### It shall provide a horizontal resolution of 480 Television lines / picture height.

### It shall be capable of providing an image at a scene illumination of 0.7 lux, using an F1.2 lens with no bubble.

### The dome shall have a 3-axis GyroView set up.

### It shall be Surface or Ceiling mountable.

### The dome shall have a removable shroud for ceiling mount purposes. Once the shroud has been removed, the dome should be secured via the integral fixing clamps.

### Fixing screws should be provided to allow the dome to be surface mounted.

### The camera shall have a signal to noise ratio of 50dB under normal operating conditions and no additional gain.

### The camera shall automatically switch from colour mode to monochrome mode when the light level reduces below a threshold level. It shall automatically switch back to colour when sufficient scene illumination is achieved.

### The camera shall have an integrated 3~9mm Varifocal lens with DC iris.

### The Electronic Iris on the camera shall have the facility to vary the shutter speed from 1/50 to 1/100,000 of a second, to enable fast capture of moving objects.

### The camera shall have settings for Back Light Compensation (BLC) - On/Off and centre weighted - and Automatic Gain Control (AGC) – On/Off, max <28dB.

### The camera shall have gamma correction of 0.45.

### The camera shall provide a standard CCIR composite video signal of 1v pk-pk when terminated with 75. Connection shall be via a 75 BNC connector.

### The camera shall have an extra video output on the front for a service connector. This optional service connector will be called ICED-SERV.

### The camera shall be capable of line locking to the external low voltage AC power supply. It shall also be capable of phase correction of ±180° using a potentiometer. The camera shall also have the facility to be internally synchronised.

### The camera shall operate when powered by either 12V DC (±10%) or 24V AC (±15%). Power consumption shall be less than 3.4 Watts.

### All settings should be situated adjacent to the lens.

### The camera shall be a Baxall ICED-CM3H39.

## 1/3" Ultra Resolution Colour/Mono Dome Camera - ICED-CM3U39

### The camera shall be a 1/3” Super HAD™ CCD with digital signal processing facilities.

### It shall provide a horizontal resolution of 540 Television lines / picture height.

### It shall be capable of providing an image at a scene illumination of 0.7 lux, using an F1.2 lens with no bubble.

### The dome shall have a 3-axis GyroView set up.

### It shall be Surface or Ceiling mountable.

### The dome shall have a removable shroud for ceiling mount purposes. Once the shroud has been removed, the dome should be secured via the integral fixing clamps.

### Fixing screws should be provided to allow the dome to be surface mounted.

### The camera shall have a signal to noise ratio of 50dB under normal operating conditions and no additional gain.

### The camera shall automatically switch from colour mode to monochrome mode when the light level reduces below a threshold level. It shall automatically switch back to colour when sufficient scene illumination is achieved.

### The camera shall have an integrated 3~9mm Varifocal lens with DC iris.

### The Electronic Iris on the camera shall have the facility to vary the shutter speed from 1/50 to 1/100,000 of a second, to enable fast capture of moving objects.

### The camera shall have settings for Back Light Compensation (BLC) - On/Off and centre weighted - and Automatic Gain Control (AGC) – On/Off, max <28dB.

### The camera shall have gamma correction of 0.45.

### The camera shall provide a standard CCIR composite video signal of 1v pk-pk when terminated with 75. Connection shall be via a 75 BNC connector.

### The camera shall have an extra video output on the front for a service connector. This optional service connector will be called ICED-SERV.

### The camera shall be capable of line locking to the external low voltage AC power supply. It shall also be capable of phase correction of ±180° using a potentiometer. The camera shall also have the facility to be internally synchronised.

### The camera shall operate when powered by either 12V DC (±10%) or 24V AC (±15%). Power consumption shall be less than 3.4 Watts.

### All settings should be situated adjacent to the lens.

### The camera shall be a Baxall ICED-CM3U39.

### Day/Night Switching Dome Cameras

## 1/3" Ultra Res Day/Night Switching Dome Camera - ICED-DN3U39

### The camera shall be a 1/3” SuperHAD™ CCD with digital signal processing facilities.

### It shall provide a horizontal resolution of 540 Television lines / picture height.

### It shall be capable of providing a colour image at a scene illumination of 0.7 lux, using an F1.6 lens with no bubble, and a monochrome image at a scene illumination of 0.1 lux using an F1.6 lens with no bubble.

### The dome shall have a 3-axis GyroView set up.

### It shall be Surface or Ceiling mountable.

### The dome shall have a removable shroud for ceiling mount purposes. Once the shroud has been removed, the dome should be secured via the integral fixing clamps.

### Fixing screws should be provided to allow the dome to be surface mounted.

### The camera shall have a signal to noise ratio of better than 50dB under normal operating conditions and no additional gain.

### The camera shall automatically switch from colour mode to monochrome mode when the light level reduces below a threshold level. It shall automatically switch back to colour when sufficient scene illumination is achieved.

### The camera shall have an integrated 3.6~9mm Varifocal lens with DC iris.

### The camera shall have a Day/Night Hi/Lo setting.

### The Electronic Iris on the camera shall have the facility to vary the shutter speed from 1/50 to 1/100,000 of a second, to enable fast capture of moving objects.

### The camera shall have settings for Back Light Compensation (BLC) - either On/Off, centre-weighted - and Automatic Gain Control (AGC) – On/Off, max <28dB.

### The camera shall have gamma correction of 0.45.

### The camera shall provide a standard CCIR composite video signal of 1v pk-pk when terminated with 75. Connection shall be via a 75 BNC connector.

### The camera shall have an extra video output on the front for a service connector. This optional service connector will be called ICED-SERV.

### The camera shall be capable of line locking to the external low voltage AC power supply. It shall also be capable of phase correction of ±180° using a potentiometer. The camera shall also have the facility to be internally synchronised.

### The camera shall operate when powered by either 12V DC (±10%) or 24V AC (±15%). Power consumption shall be less than 3.4 Watts.

### All settings and controls should be situated adjacent to the lens.

### The camera shall be a Baxall ICED-DN3U39.

## Wide Dynamic Range Dome Cameras

## Wide Dynamic Colour Dome Camera - ICED-HYPERDOME

### The camera shall use a Digital Image Sensor.

### It shall provide a horizontal resolution of 480 Television lines and a vertical resolution of 400 Television lines.

### It shall be capable of providing an image at a scene illumination of 1.0 lux, using an F1.2 lens with no bubble.

### The dome shall have a 3-axis GyroView set up.

### It shall be Surface or Ceiling mountable.

### The dome shall have a removable shroud for ceiling mount purposes. Once the shroud has been removed, the dome should be secured via the integral fixing clamps.

### Fixing screws should be provided to allow the dome to be surface mounted.

### The camera shall have a signal to noise ratio of better than 48dB under normal operating conditions and no additional gain.

### The camera shall use Hyper D technology and have zero smear.

### The camera shall have an integrated 3~9mm Varifocal lens with DC iris.

### The camera shall have Automatic Back Light Compensation (BLC).

### It shall have digital Automatic Gain Control (AGC).

### The camera shall have gamma correction of 0.45.

### The camera shall provide a standard CCIR composite video signal of 1v pk-pk when terminated with 75. Connection shall be via a 75 BNC connector.

### The camera shall have an extra video output on the front for a service connector. This optional service connector will be called ICED-SERV.

### The camera shall be capable of line locking to the external low voltage AC power supply. It shall also be capable of phase correction of ±360° using a potentiometer. The camera shall also have the facility to be internally synchronised.

### The camera shall operate when powered by either 12V DC (±10%) or 24V AC (±15%). Power consumption shall be less than 3.4 Watts.

### All settings and controls should be situated adjacent to the lens.

### The camera shall be a Baxall ICED-HYPERDOME

.