# Upgrading PY2/3 keyboards

1.	Run the Teraterm Winzip. (Click on the setup.exe). This will install Teraterm

2.	Copy the PY2KEYBF.s19 file to a directory on your PC

3.	Run Teraterm from Start menu

Go to Set-up menu, select Serial port

Set to COM1, baud = 38,400, 8 Data bits, Parity none, Stop 1, Flow none

Click OK

### PY2 Keyboard

4.	Uncover the base plate from the PY2 keyboard (2 screws).

- Insert Serial lead (same one as you use with the PY2-INS (SLTA).

### PY3 Keyboard

- Connect serial data connections to the PY-NAP in the following manner – Pin 2 (TX) to RED, Pin 3 (RX) to Green and Pin 5 (GND) to Blue.

### Both types of keyboard

7.	Connect the other end of the serial lead to the Com port of PC.

7.	Power up keyboard, whilst pressing the centre orange button down.

8.	Teraterm should show the boot up screen and text showing

## “Forced download, waiting for data”

9.	Click on File | Send File

10.	Navigate to where you stored the software file and click on file

11.	Ensure Binary box at base of window is ticked

12.	Click Open button

13.	Rotating / \ should be shown on LCD display

14.	Download should take approx. 10 minutes after which the keyboard will reboot.